[ EMBEDDED C PROGRAMMING ASSIGNMENT ]

NAME :- SHUBHAM PRAJAPATI

COURSE :- BTECH ( ELECTRICAL ENGINEERING )

SUBJECT :- PROGRAMMING FUNDAMENTALS FOR ELECTRONICS ENGINEERS

ADMISSION NUMBER :- 25SECE1050013

- Write a Program to set(enable ) the 5th bit of a it register .

#code

#include <stdio.h>

int main() {

int reg;

printf("Enter register value: ");

scanf("%d", &reg);

reg = reg | (1 << 4);

printf("Value after setting 5th bit: %d\n", reg);

return 0;

}

- Write a program to clear the 2nd bit of a 16- bit register .

#code

#include <stdio.h>

int main() {

unsigned short reg;

printf("Enter register value: ");

scanf("%hu", &reg)

reg = reg & ~(1 << 1);

printf("Value after clearing 2nd bit: %hu\n", reg);

return 0;

}

3.Write a program to toggle the 7th bit of an 8 bit register.

#code

#include <stdio.h>

int main() {

unsigned char reg;

printf("Enter register value: ");

scanf("%hhu", &reg);

reg = reg ^ (1 << 6);

printf("Value after toggling 7th bit: %hhu\n", reg);

return 0;

}

4.Write a program to check whether a given bit position is set or not .(4th bit )

#code

#include <stdio.h>

int main() {

int num;

printf("Enter a number: ");

scanf("%d", &num);

if (num & (1 << 3)) {

printf("4th bit is SET\n");

} else {

printf("4th bit is NOT SET\n");

}

return 0;

}

5.Write a program to count the number of set bits(1s) in a number.

#code

#include <stdio.h>

int main() {

int num, count = 0;

printf("Enter a number: ");

scanf("%d", &num);

while (num > 0) {

if (num & 1) {

count++;

}

num = num >> 1;

}

printf("Number of set bits = %d\n", count);

return 0;

}

6. Write a program to perform left shift and right shift operations on a number and display result .

#code

#include <stdio.h>

int main() {

int num, leftShift, rightShift;

printf("Enter a number: ");

scanf("%d", &num);

leftShift = num << 1;

rightShift = num >> 1;

printf("After Left Shift by 1: %d\n", leftShift);

printf("After Right Shift by 1: %d\n", rightShift);

return 0;

}

7. Write a program to swap two numbers using bitwise XOR.

#code

#include <stdio.h>

int main() {

int a, b;

printf("Enter first number: ");

scanf("%d", &a);

printf("Enter second number: ");

scanf("%d", &b);

printf("\nBefore swapping:\n");

printf("a = %d\nb = %d\n", a, b);

a = a ^ b;

b = a ^ b;

a = a ^ b;

printf("\nAfter swapping:\n");

printf("a = %d\nb = %d\n", a, b);

return 0;

}

8. Write a program to clear all bits except a specific bit position.

#code

#include <stdio.h>

int main() {

int num, pos, result;

printf("Enter a number: ");

scanf("%d", &num);

printf("Enter bit position to keep (0-based index): ");

scanf("%d", &pos);

result = num & (1 << pos);

printf("Result after clearing all bits except position %d = %d\n", pos, result);

return 0;

}

9. Write a program to blink a led connected to pin number 2 of a microcontroller. Use software delay.

#code

#include <REGX51.H>

sbit LED = P2^0;

void delay() {

unsigned int i, j;

for(i = 0; i < 1000; i++) {

for(j = 0; j < 100; j++);

}

}

void main() {

while(1) {

LED = 0;

delay();

LED = 1;

delay();

}

}

10. Write an embedded C program to check the temperature. If it is more than 35 degree Centigrade, send high signal to fan so that it can turn on.

#code

#include <REGX51.H>

sbit FAN = P2^0;

void main()

{

unsigned char temperature;

while(1)

{

temperature = P1;

if(temperature > 35)

{

FAN = 1;

}

else

{

FAN = 0;

}

}

}

#Output